# Section 1: Identification

Product Code:	FR0912

Product Name:	Farm and Ranch Gloss Tractor Green

Product Family:	Oil-Base Alkyd Paint

Recommended Use:	Paint for architectural and industrial maintenance applications.

Company Information:	Customer Service:

Anchor Paint Manufacturing Co., Inc.	Phone: 918-836-4626

6707 East 14th Street	Fax: 918-836-6421

Tulsa, OK 74112

800-999-4626

Emergency Information:

CHEMTREC	800-424-9300

Poison Control	800-222-1222

# Section 2: Hazards Identification

GHS Classifications

Acute Aquatic Toxicity Category 3

Eye Damage/Irritation, Category 2A

Flammable Liquid, Category 3

Eye Damage/Irritation Category 2A

Flammable Solid - Category 2

Germ cell mutagenicity Category 1B

Reproductive Toxicity Category 2

Specific target organ toxicity (repeated exposure) Category 1

Skin Irritation Category 3

Label Elements

Hazard Pictograms:

## Signal Word: Danger

Hazard Statements:

Flammable liquid and vapor.

Causes skin irritation.

Causes serious eye irritation.

May cause genetic defects.

May cause cancer.

May damage fertility or the unborn child.

Causes damage to organs through prolonged or repeated exposure

Toxic to aquatic life with long lasting effects

Harmful to aquatic life with long lasting effects

Full text of H-phrases: see section 16

## Precautionary Statements:

If medical advice is needed, have product container or label at hand.

Keep out of reach of children.

Read label carefully and follow all instructions

Obtain special instructions before use.

Do not handle until all safety precautions have been read and understood

Keep away from sparks, open flames, and hot surfaces. No smoking.

Keep container tightly closed.

Ground container and receiving equipment.

Use explosion-proof equipment.

Use only non-sparking tools.

Take precautionary measures against static discharge

Do not breathe vapors or spray mist.

Wash thoroughly after handling.

Do not eat, drink, or smoke when using this product.

Avoid release to the environment.

Use protective gloves, clothing and eye protection.

IF ON SKIN (or hair): Remove immediately all contaminated clothing and wash with plenty of soap and water.

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

IF exposed or concerned: get medical advice.

Get medical advice/attention if you feel unwell.

If skin irritation occurs: Get medical advice or attention.

In case of fire: Use CO2, dry chemical or foam for extinction.

Collect spillage.

Store locked up in a well-ventilated place. Keep cool.

Dispose of contents to an approved waste disposal plant or paint recycling center.

Other Hazards:

NOTICE: Reports have associated repeated and prolonged occupational overexposure to solvents with permanent brain and nervous system damage. Intentional misuse by deliberately concentrating and inhaling the contents may be harmful or fatal.

Acute Toxicity of 41% of mixture is unknown.

# Section 3: Composition/Information on Ingredients

| Name | CAS No. | Approx % by Weight |
| --- | --- | --- |
| n-Butyl Acetate | 123-86-4 | 5% - 10% |
| Pigment Green 7 | 1328-53-6 | 1% - 2% |
| Xylene (o-, m-, p- isomers) | 1330-20-7 | 2% - 5% |
| Iron Hydroxide Oxide | 20344-49-4 | 2% - 5% |
| Pigment Yellow 74 | 6358-31-2 | 1% - 2% |
| Petroleum naphtha, light aromatic | 64742-95-6 | 5% - 10% |
| Aliphatic Hydrocarbon Stoddard Type | 8052-41-3 | 15% - 20% |

Identity and/or exact concentration of some components withheld due to confidentiality.

# Section 4: First Aid Measures

## Eye Contact

Immediately flush eyes with plenty of water, occasionally lifting the upper and lower eyelids. Check for and remove any contact lenses. Continue to rinse for at least 10 minutes. Get medical attention.

## Inhalation

Avoid fume exposure of the rescuer. Remove victim to fresh air and keep at rest in a position comfortable for breathing. If unconscious, place in recovery position and get medical attention immediately. Maintain an open airway. Loosen tight clothing such as a collar, tie, belt or waistband. If breathing has stopped, provide artificial respiration or use mouth-to-mouth resuscitation. If symptoms persist, get medical attention.

## Skin Contact

Wash with plenty of soap and water. Remove contaminated clothing and shoes. Continue to rinse for at least 10 minutes. If irritation occurs, get medical attention. In the event of any complaints or symptoms, avoid further exposure. Wash clothing before reuse. Clean shoes thoroughly before reuse.

## Ingestion

Get medical attention immediately. Call a poison center or physician. Wash out mouth with water. Remove dentures if any. Remove victim to fresh air and keep at rest in a position comfortable for breathing. If material has been swallowed and the exposed person is conscious, give small quantities of water to drink. Stop if the exposed person feels sick as vomiting may be dangerous. Aspiration hazard if swallowed. Can enter lungs and cause

damage. Do not induce vomiting. If vomiting occurs, the head should be kept low so that vomit does not enter the lungs. Never give anything by mouth to an unconscious person. If unconscious, place in recovery position and get medical attention immediately. Maintain an open airway. Loosen tight clothing.

# Section 5: Fire-Fighting Measures

## Suitable Extinguishing Media

Use water fog, foam, dry chemical or carbon dioxide (CO2) to extinguish.

## Unsuitable Extinguishing Media

Do not use straight streams of water.

## Specific hazards arising from the chemical

Flammable liquid and vapor. Pressure may build and cause rupture in heated containers. Vapor is heavier than air and will spread along the ground.

## Hazardous Combustion Products

Oxides of carbon, metal oxides.

## Special Protective Equipment and Precautions for Firefighters

No action shall be taken involving any personal risk or without suitable training. Firefighters should use standard protective equipment and in enclosed spaces, self-containing breathing apparatus (SCBA).

## Additional Firefighting Instructions

Evacuate surrounding areas. Use water spray to keep fire-expose containers cool. Do not allow run-off from fire fighting to enter drains. Collect contaminated fire extinguishing water and dispose of in accordance with local regulations.

# Section 6: Accidental Release Measures

## Personal Precautions, Protective Equipment, and Emergency Procedures

Avoid breathing vapors. Remove all sources of ignition and avoid static electricity discharges. Use protective equipment as required (See Section 8). Prevent further leakage or spillage if safe to do so. Avoid runoff into storm sewers and ditches which lead to waterways. Keep unauthorized personnel from the area.

## Methods and Materials for Containment and Cleaning Up

Dam up and soak up with inert absorbent material (floor-dry, PIG absorbents, sand, or sawdust). Scoop up and transfer to properly labeled containers. Allow used absorbent material to dry and dispose of according to local regulations.

Advise local authorities if spillage cannot be contained.

# Section 7 Handling and Storage

## Precautions for Safe Handling

Prevent the creation of flammable or explosive concentrations of vapor and avoid vapor concentrations higher than the occupational exposure limits. Bond and ground metal containers when transferring flammable liquids. Avoid free fall of liquid in excess of a few inches. Personal contact should be avoided by using the personal protective equipment listed in Section 8.

## Conditions for Safe Storage

Keep containers cool, dry, and away from sources of ignition. Store in a well-ventilated area and prevent unauthorized access.

## Emptied Containers

Emptied containers may retain product residue and retain all hazards. Do not cut, weld, drill, grind, or expose empty containers to other sources of ignition. Doing so may cause explosion, injury, or death.

## Incompatibilities

Strong oxidizers. Ignition sources.

# Section 8: Exposure Controls/Personal Protection

| Name | OSHA PEL (TWA)(mg/m3) | OSHA PEL (TWA)(ppm) |
| --- | --- | --- |
| n-Butyl Acetate (123-86-4) | TWA: 710 mg/m3 | TWA: 150 ppm |
| Pigment Green 7 (1328-53-6) | No Data Available | No Data Available |
| Xylene (o-, m-, p- isomers) (1330-20-7) | TWA: 435 mg/m3 | TWA: 100 ppm |
| Iron Hydroxide Oxide (20344-49-4) | No Data Available | No Data Available |
| Pigment Yellow 74 (6358-31-2) | No Data Available | No Data Available |
| Petroleum naphtha, light aromatic (64742-95-6) |  |  |
| Aliphatic Hydrocarbon Stoddard Type (8052-41-3) | PEL TWA: 2900 mg/m3 | PEL TWA: 500 ppm |

## Respiratory Protection

Keep area well-ventilated insuring that TLV or PELS are not exceeded. Use forced air movement such as spray booths or fans. If exposure exceeds TLV, PELS, or if personnel experience inhalation irritation, use a NIOSH approved respirator. If a respirator is needed, current engineering controls are not adequate and should be re-evaluated.

## Skin Protection

Skin protection is required for prolonged or repeated contact. Wear chemical resistant gloves and imperious

clothing. Liquid may penetrate shoes and other clothing causing delayed irritation. Remove contaminated clothing as soon as possible and wash hands before eating, smoking or using the restroom.

## Eye Protection

Use safety eyewear designed to protect against splash of liquids and vapors.

## Other Equipment

Emergency eye wash capability and the ability to wash with soap and water should be readily available. Use NIOSH-approved air-purifying or supplied-air respirator where airborne concentrations of vapor or mist are expected to exceed exposure limits.

# Section 9: Physical and Chemical Properties

| Appearance | Opaque Liquid |
| --- | --- |
| Odor | Petroleum Solvent |
| Odor Threshold | No Information Available |
| pH | Not Applicatable |
| Melting/Freezing Point | No Information Available |
| Boiling Point (Point/Range) | 316 °F - 399 °F |
| Flash Point | 80 °F |
| Evaporation Rate | 0.14 |
| Flammability | Flammable Liquid |
| UEL | Approximate 5.6 |
| LEL | Approximate 0.7 |
| Vapor Pressure | No Information Available |

| Vapor Density | Heavier Than Air |
| --- | --- |
| Relative Density | 1.02661 |
| Solubility | Most Solvents, Not Water |
| Partition Coefficient: n-octanal/water | No Information Available |
| Auto-ignition Temperature | No Information Available |
| Decomposition Temperature | No Information Available |
| Viscosity | 100-500 cSt (mm2/sec) @ 40 °C |
| Regulatory VOC: | 361.72779 g/l (3.01876 lb/gal) |
| Actual VOC: | 361.72779 g/l (3.01876 lb/gal) |
| % Solids By Weight: | 64.66431 |
| % Solid By Volume: | 55.70151 |

Section 10: Stability and Reactivity

## Reactivity

No dangerous reaction is known under conditions of normal use.

## Chemical Stability

This product is stable.

## Possibility of Hazardous Reactions

Spontaneous combustion of paint-soaked rags, spray booth filters, or dry-spray floor sweepings.

## Conditions to Avoid

Avoid all possible sources of ignition. Do not allow vapor to accumulate in low or confined areas. Do not pile or accumulate paint-soaked rags, filters or floor sweeping until paint is cured.

## Incompatible Materials

Strong oxidizers.

## Hazardous Decomposition Products

Under normal conditions of storage and use, hazardous decomposition should not occur.

# Section 11: Toxicological Information

Likely Routes of Exposure: Inhalation, Ingestion, Skin and Eye Contact.

## Inhalation Symptoms/Effects:

May be irritating to the respiratory tract, may cause headaches, dizziness, anesthesia, drowsiness, unconsciousness and other central nervous system effects including death.

## Ingestion Symptoms/Effects:

If swallowed, can cause gastrointestinal irritation, nausea, vomiting, and diarrhea. Aspiration of material into the lungs can cause chemical pneumonitis which can be fatal.

## Skin Contact Symptoms/Effects:

Prolonged or repeated skin contact may defat the skin resulting in possible irritation and dermatitis.

## Delayed or Chronic Effects:

None known.

Ingredient Toxicity Information:

| CAS No./Test | Test Results |
| --- | --- |
| n-Butyl Acetate (123-86-4) |  |
| LC50 - Inhalation Rat | 390 ppm 4 h |
| LD50 - Oral Rat | 14.13 mg/kg |
| LD50 - Dermal Rabbit | >17,600 mg/kg |
| Pigment Green 7 (1328-53-6) |  |
| LC50 - Inhalation Rat | No Data Available |

| LD50 - Oral Rat | >2,000 mg/kg |
| --- | --- |
| LD50 - Dermal Rabbit | No Data Available |
| Xylene (o-, m-, p- isomers) (1330-20-7) |  |
| LC50 - Inhalation Rat | 6,700 ppm 4 h |
| LD50 - Oral Rat | 3,523 mg/kg |
| LD50 - Dermal Rabbit | 1,100 mg/kg |
| Iron Hydroxide Oxide (20344-49-4) |  |
| LC50 - Inhalation Rat | No Data Available |
| LD50 - Oral Rat | > 5,000 mg/kg |
| LD50 - Dermal Rabbit | Non-irritating |
| Pigment Yellow 74 (6358-31-2) |  |
| LC50 - Inhalation Rat | No Data Available |
| LD50 - Oral Rat | >5,000 mg/kg |
| LD50 - Dermal Rabbit | No Data Available |
| Petroleum naphtha, light aromatic (64742-95-6) |  |
| LC50 - Inhalation Rat | 3,400 ppm 4 h |
| LD50 - Dermal Rabbit | >2,000 mg/kg |
| Aliphatic Hydrocarbon Stoddard Type (8052-41-3) |  |
| LC50 - Inhalation Rat | >13.1 mg/l 4 h |
| LD50 - Oral Rat | >15,000 mg/kg |
| LD50 - Dermal Rabbit | >3,400 mg/kg |

## Ingredients with Potential Carcinogenicity:

No known ingredients are classified as carcinogens.

Section 12: Ecological Information

## Ecotoxicity:

Some ingredients are harmful to aquatic life with long lasting effects. Prevent product from entering drains.

## Persistence and Degradability:

No information available.

## Bioaccumulation:

No information available.

## Mobility:

Air - Liquid components are volatile and will partition to air. Soil - No information available.

## Other Adverse Effects:

No information available.

# Section 13: Disposal Considerations

## Product Stewardship:

The generation of waste should be avoided or minimized wherever possible. Use the spreading rate recommendation in the Product Data Sheet and job size to estimate needed quantities before acquiring product.

## Waste Disposal:

Dispose in accordance with all applicable federal, state, and local regulations. Recycle if a recycling center is available that accepts oil-based alkyd paints.

## Emptied Containers:

Emptied containers may retain product residue and retain hazards. Do not cut, weld, drill, grind, or expose empty containers to other sources of ignition. Doing so may cause explosion, injury, or death. Allow containers to dry thoroughly before disposal.

# Section 14: Transport Information

## Land US Domestic (DOT)

UN Number:	UN1263

UN Proper Shipping Name:	Paint

Transport Hazard Class:	III

Environmental Hazard:	No

Hazardous Substance:	No

Container sizes one gallon or less qualify for Limited Quantity exceptions (See 49 CFR 173.150).

# Section 15: Regulatory Information

## State and Local Regulations

Xylene (o-, m-, p- isomers) (1330-20-7)

U.S. California Proposition 65 Carcinogens List: WARNING this product contains chemicals known to the state of California to cause cancer.

Federal Regulations

n-Butyl Acetate (123-86-4)

VOC Reportable

CERCLA 40 CFR 302.4

SARA 311/312 Hazardous

Listed on the United States TSCA (Toxic Substances Control Act) inventory

Xylene (o-, m-, p- isomers) (1330-20-7)

VOC Reportable

CERCLA 40 CFR 302.4

IARC_Carcinogen

RCRA Reportable

SARA 311/312 Hazardous

SARA Section 313 - Emission Reporting: 1.0%

Listed on the United States TSCA (Toxic Substances Control Act) inventory

Petroleum naphtha, light aromatic (64742-95-6)

VOC Reportable

SARA 311/312 Hazardous

Listed on the United States TSCA (Toxic Substances Control Act) inventory

Aliphatic Hydrocarbon Stoddard Type (8052-41-3)

VOC Reportable

SARA 311/312 Hazardous

Listed on the United States TSCA (Toxic Substances Control Act) inventory

SARA 311/312 Hazard Categories: Fire, Acute Health, Chronic Health

## California Proposition 65:

This product contains chemicals known to the State of California to cause cancer and/or to cause birth defects or other reproductive harm.

# Section 16: Other Information

GHS Full text phrases

| Code/Abbreviation | Description |
| --- | --- |
| H226 | Flammable liquid and vapor. |
| H315 | Causes skin irritation. |
| H319 | Causes serious eye irritation. |
| H340 | May cause genetic defects. |
| H350 | May cause cancer. |
| H360 | May damage fertility or the unborn child. |
| H372 | Causes damage to organs through prolonged or repeated exposure |
| H411 | Toxic to aquatic life with long lasting effects |
| H412 | Harmful to aquatic life with long lasting effects |

This document has been prepared in accordance with the SDS requirements of the OSHA Hazard Communication Standard 29 CFR 1910.1200.

This information is furnished without warranty, representation, inducement or license of any kind, except that it is accurate to the best of Anchor Paint Manufacturing Co., Inc. knowledge or obtained from sources believed by Anchor Paint Manufacturing Co., Inc. to be accurate and Anchor Paint Manufacturing Co., Inc. does not assume any legal responsibility for use or reliance upon same. Customers are encouraged to conduct their own tests.