# Section 1: Identification

Product Code:	FR1100

Product Name:	Polyurethane Converter

Product Family:	Additives

Recommended Use:	Polyurethane modifier for alkyd paints

### Company Information:	Customer Service:

Anchor Paint Manufacturing Co., Inc.	Phone: 918-836-4626

6707 East 14th Street	Fax: 918-836-6421

Tulsa, OK 74112

800-999-4626

### Emergency Information:

CHEMTREC	800-424-9300

Poison Control	800-222-1222

# Section 2: Hazards Identification

## GHS Classifications

Acute Aquatic Toxicity Category 3

Chronic aquatic toxicity - Category 3

Skin Corrosion/Irritation, Category 2

Carcinogenicity, Category 2

Specific Target Organ Toxicity (Single Exposure)(Respiratory Track Irritation), Category 3

Eye Damage/Irritation Category 2B

Flammable liquid - Category 3

Respiratory Sensitizer (Solid/Liquid) - Category 1

Skin sensitization Category 1

Label Elements

### Hazard Pictograms:

Signal Word:	Warning

Hazard Statements:

Flammable liquid and vapor.

Causes skin irritation.

May cause an allergic skin reaction.

Causes eye irritation.

May cause allergy or asthma symptoms or breathing difficulties if inhaled.

May cause respiratory irritation.

Suspected of causing cancer.

Harmful to aquatic life.

Harmful to aquatic life with long lasting effects

Precautionary Statements:

If medical advice is needed, have product container or label at hand.

Keep out of reach of children.

Read label carefully and follow all instructions

Obtain special instructions before use.

Do not handle until all safety precautions have been read and understood

Keep away from sparks, open flames, and hot surfaces. No smoking.

Keep container tightly closed.

Ground container and receiving equipment.

Use explosion-proof equipment.

Use only non-sparking tools.

Take precautionary measures against static discharge

Avoid breathing dust/fume/gas/mist/vapor/spray

Wash thoroughly after handling.

Use only outdoors or in a well-ventilated area.

Contaminated work clothing should not be allowed out of the workplace.

Avoid release to the environment.

Use protective gloves, clothing and eye protection.

In case of inadequate ventilation, wear respiratory protection.

IF ON SKIN (or hair): Rinse skin with water.

IF ON SKIN (or hair): Remove immediately all contaminated clothing and wash with plenty of soap and water.

IF INHALED: Remove victim to fresh air and keep at rest in a position comfortable for breathing.

IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing.

IF exposed or concerned: get medical advice.

Call a POISON CENTER or doctor if you feel unwell.

Specific Treatment (See Section 4 on this SDS)

If skin irritation occurs: Get medical advice or attention.

If skin irritation or rash occurs: Get medical advice/attention

If eye irritation persists: Get medical attention.

Take off contaminated clothing and wash before reuse.

In case of fire: Use CO2, dry chemical or foam for extinction.

Store locked up in a well-ventilated place. Keep cool.

Dispose of contents to an approved waste disposal plant or paint recycling center.

Other Hazards:

NOTICE: Reports have associated repeated and prolonged occupational overexposure to solvents with permanent brain and nervous system damage. Intentional misuse by deliberately concentrating and inhaling the contents may be harmful or fatal.

# Section 3: Composition/Information on Ingredients

| Name | CAS No. | Approx % by Weight |
| --- | --- | --- |
| Xylene (o-, m-, p- isomers) | 1330-20-7 | < 1% |
| Isophorone Diisocyanate Homopolymer | 53880-05-0 | 65% - 70% |
| Petroleum naphtha, light aromatic | 64742-95-6 | 20% - 25% |
| 1,2,4-Trimethylbenzene | 95-63-6 | 2% - 5% |

Any concentration shown as a range is to protect confidentiality or is due to batch variation. Full text of H-phrases: see section 16

# Section 4: First Aid Measures

## Eye Contact

Immediately flush eyes with plenty of water, occasionally lifting the upper and lower eyelids. Check for and remove any contact lenses. Continue to rinse for at least 10 minutes. Get medical attention.

## Inhalation

Avoid fume exposure of the rescuer. Remove victim to fresh air and keep at rest in a position comfortable for breathing. If unconscious, place in recovery position and get medical attention immediately. Maintain an open airway. Loosen tight clothing such as a collar, tie, belt or waistband. If breathing has stopped, provide artificial respiration or use mouth-to-mouth resuscitation. If symptoms persist, get medical attention.

## Skin Contact

Wash with plenty of soap and water. Remove contaminated clothing and shoes. Continue to rinse for at least 10 minutes. If irritation or rash occurs, get medical attention. In the event of any complaints or symptoms, avoid further exposure. Wash clothing before reuse. Clean shoes thoroughly before reuse.

## Ingestion

Get medical attention immediately. Call a poison center or physician. Wash out mouth with water. Remove dentures if any. Remove victim to fresh air and keep at rest in a position comfortable for breathing. If material has been swallowed and the exposed person is conscious, give small quantities of water to drink. Stop if the exposed person feels sick as vomiting may be dangerous. Aspiration hazard if swallowed. Can enter lungs and cause

damage. Do not induce vomiting. If vomiting occurs, the head should be kept low so that vomit does not enter the lungs. Never give anything by mouth to an unconscious person. If unconscious, place in recovery position and get medical attention immediately. Maintain an open airway. Loosen tight clothing.

# Section 5: Fire-Fighting Measures

### Suitable Extinguishing Media

Use water fog, foam, dry chemical or carbon dioxide (CO2) to extinguish.

### Unsuitable Extinguishing Media

Do not use straight streams of water.

### Specific hazards arising from the chemical

Flammable liquid and vapor. Pressure may build and cause rupture in heated containers. Vapor is heavier than air and will spread along the ground.

### Hazardous Combustion Products

Oxides of carbon, metal oxides.

### Special Protective Equipment and Precautions for Firefighters

No action shall be taken involving any personal risk or without suitable training. Firefighters should use standard protective equipment and in enclosed spaces, self-containing breathing apparatus (SCBA).

### Additional Firefighting Instructions

Evacuate surrounding areas. Use water spray to keep fire-expose containers cool. Do not allow run-off from fire fighting to enter drains. Collect contaminated fire extinguishing water and dispose of in accordance with local regulations.

# Section 6: Accidental Release Measures

### Personal Precautions, Protective Equipment, and Emergency Procedures

Avoid breathing vapors. Remove all sources of ignition and avoid static electricity discharges. Use protective equipment as required (See Section 8). Prevent further leakage or spillage if safe to do so. Avoid runoff into storm sewers and ditches which lead to waterways. Keep unauthorized personnel form the area.

### Methods and Materials for Containment and Cleaning Up

Dam up and soak up with inert absorbent material (floor-dry, PIG absorbents, sand, or sawdust). Scoop up and transfer to properly labeled containers. Allow used absorbent material to dry and dispose of according to local regulations.

Advise local authorities if spillage cannot be contained.

# Section 7 Handling and Storage

### Precautions for Safe Handling

Prevent the creation of flammable or explosive concentrations of vapor and avoid vapor concentrations higher than the occupational exposure limits. Bond and ground metal containers when transferring flammable liquids. Avoid free fall of liquid in excess of a few inches. Personal contact should be avoided by using the personal protective equipment listed in Section 8.

### Conditions for Safe Storage

Keep containers cool, dry, and away from sources of ignition. Store in well-ventilated area and prevent unauthorized access.

### Emptied Containers

Emptied containers may retain product residue and retain all hazards. Do not cut, weld, drill, grind, or expose empty containers to other sources of ignition. Doing so may cause explosion, injury, or death.

### Incompatibilities

Strong oxidizers. Ignition sources.

# Section 8: Exposure Controls/Personal Protection

| Name | OSHA PEL (TWA)(mg/m3) | OSHA PEL (TWA)(ppm) |
| --- | --- | --- |
| Xylene (o-, m-, p- isomers) (1330-20-7) | TWA: 435 mg/m3 | TWA: 100 ppm |
| Isophorone Diisocyanate Homopolymer (53880-05-0) |  |  |
| Petroleum naphtha, light aromatic (64742-95-6) |  |  |
| 1,2,4-Trimethylbenzene (95-63-6) | TWA: 125 mg/m3 | TWA: 25 ppm |

### Respiratory Protection

Keep area well ventilated insuring that TLV or PELS are not exceeded. Use forced air movement such as spray booths or fans. If exposure exceeds TLV, PELS, or if personnel experience inhalation irritation, use a NIOSH approved respirator. If a respirator is needed, current engineering controls are not adequate and should be re-evaluated.

### Skin Protection

Skin protection is required for prolonged or repeated contact. Wear chemical resistant gloves and imperious

clothing. Liquid may penetrate shoes and other clothing causing delayed irritation. Remove contaminated clothing as soon as possible and wash hands before eating, smoking or using the restroom.

### Eye Protection

Use safety eyewear designed to protect against splash of liquids and vapors.

### Other Equipment

Emergency eye wash capability and the ability to wash with soap and water should be readily available. Use NIOSH-approved air-purifying or supplied-air respirator where airborne concentrations of vapor or mist are expected to exceed exposure limits.

Other information: When using, do not eat, drink or smoke.

# Section 9: Physical and Chemical Properties

| Appearance | Clear Liquid |
| --- | --- |
| Odor | Solvent |
| Odor Threshold | No Information Available. |
| pH | No Information Available. |
| Melting/Freezing Point | No Information Available. |
| Boiling Point (Point/Range) | No Information Available. |
| Flash Point | 113F |
| Evaporation Rate | No Information Available. |
| Flammability | Flashpoint at or above 100F but less than 200F |
| UEL | 7.5 |
| LEL | 1 |
| Vapor Pressure | No Information Available. |

| Vapor Density | Heavier than air |
| --- | --- |
| Relative Density | 1.12845 |
| Solubility | No Information Available. |
| Partition Coefficient: n-octanal/water | No Information Available. |
| Auto-ignition Temperature | 450F |
| Decomposition Temperature | No Information Available. |
| Viscosity | No Information Available. |
| Regulatory VOC: | 337.91053 g/l (2.82000 lb/gal) |
| Actual VOC: | 337.91053 g/l (2.82000 lb/gal) |
| % Solids By Weight: | 70.00000 |
| % Solid By Volume: | 59.49367 |

Section 10: Stability and Reactivity

### Reactivity

No dangerous reaction known under conditions of normal use.

### Chemical Stability

This product is stable.

### Possibility of Hazardous Reactions

Spontaneous combustion of paint-soaked rags, spray booth filters, or dry-spray floor sweepings.

### Conditions to Avoid

Avoid all possible sources of ignition. Do not allow vapor to accumulate in low or confined areas. Do not pile or accumulate paint-soaked rags, filters or floor sweeping until paint is cured.

### Incompatible Materials

Strong oxidizers.

### Hazardous Decomposition Products

Under normal conditions of storage and use, hazardous decomposition should not occur.

# Section 11: Toxicological Information

### Information on toxicological effects:

Likely Routes of Exposure: Inhalation, Ingestion, Skin and Eye Contact.

### Inhalation Symptoms/Effects:

May be irritating to the respiratory tract, may cause headaches, dizziness, anesthesia, drowsiness, unconsciousness and other central nervous system effects including death.

### Ingestion Symptoms/Effects:

If swallowed, can cause gastrointestinal irritation, nausea, vomiting, and diarrhea. Aspiration of material into the lungs can cause chemical pneumonitis which can be fatal.

### Skin Contact Symptoms/Effects:

Prolonged or repeated skin contact may defat the skin resulting in possible irritation and dermatitis.

### Delayed or Chronic Effects:

May cause damage to organs through prolonged or repeated exposure.

Ingredient Toxicity Information:

| CAS No./Test | Test Results |
| --- | --- |
| Xylene (o-, m-, p- isomers) (1330-20-7) |  |
| LC50 - Inhalation Rat | 6,700 ppm 4 h |
| LD50 - Oral Rat | 3,523 mg/kg |
| LD50 - Dermal Rabbit | 1,100 mg/kg |
| Isophorone Diisocyanate Homopolymer (53880-05-0) |  |
| LC50 - Inhalation Rat | > 5 mg/l |
| LD50 - Oral Rat | 14,000 mg/kg |
| Petroleum naphtha, light aromatic (64742-95-6) |  |

| LC50 - Inhalation Rat | 3,400 ppm 4 h |
| --- | --- |
| LD50 - Dermal Rabbit | >2,000 mg/kg |
| 1,2,4-Trimethylbenzene (95-63-6) |  |
| LC50 - Inhalation Rat | 18 g/m3 4 h |
| LD50 - Oral Rat | 3280 mg/kg |
| LD50 - Dermal Rabbit | >3160 mg/kg |

Ingredients with Potential Carcinogenicity:

No known ingredients are classified as carcinogens.

## Section 12: Ecological Information

### Ecotoxicity:

Some ingredients are harmful to aquatic life with long lasting effects. Prevent product from entering drains.

### Persistence and Degradability:

No information available.

### Bioaccumulation:

No information available.

### Mobility:

Air - Liquid components are volatile and will partition to air. Soil - No information available.

### Other Adverse Effects:

No information available.

# Section 13: Disposal Considerations

### Product Stewardship:

The generation of waste should be avoided or minimized wherever possible. Use the spreading rate recommendation in the Product Data Sheet and job size to estimate needed quantities before acquiring product.

### Waste Disposal:

Dispose in accordance with all applicable federal, state, and local regulations. Recycle if a recycling center is available that accepts oil-based alkyd paints.

### Emptied Containers:

Emptied containers may retain product residue and retain hazards. Do not cut, weld, drill, grind, or expose empty containers to other sources of ignition. Doing so may cause explosion, injury, or death. Allow containers to dry thoroughly before disposal.

# Section 14: Transport Information

### Land US Domestic (DOT)

UN Number:	UN1263

UN Proper Shipping Name:	Paint

Transport Hazard Class:	III

Environmental Hazard:	No

Hazardous Substance:	No

For ground transport this product may be re-classed as a combustible liquid and is not regulated as hazardous material in non-bulk packages (See 49 CFR 173.150(f)).

# Section 15: Regulatory Information

SARA 311/312 Hazard Categories: Fire, Acute Health, and Chronic Health

SARA (313) Toxic Chemicals

All ingredients are listed or exempt from listing on the following inventories: TSCA, DLS

California Proposition 65:

This product contains chemicals known to the State of California to cause cancer and/or to cause birth defects or other reproductive harm.

State Regulations

Xylene (o-, m-, p- isomers) (1330-20-7)

U.S. California Proposition 65 Carcinogens List: WARNING this product contains chemicals known to the state of California to cause cancer.

Federal Regulations

Xylene (o-, m-, p- isomers) (1330-20-7)

VOC Reportable

CERCLA 40 CFR 302.4

IARC_Carcinogen

RCRA Reportable

SARA 311/312 Hazardous

SARA Section 313 - Emission Reporting: 1.0%

Listed on the United States TSCA (Toxic Substances Control Act) inventory

Isophorone Diisocyanate Homopolymer (53880-05-0)

SARA 311/312 Hazardous

Listed on the United States TSCA (Toxic Substances Control Act) inventory

Petroleum naphtha, light aromatic (64742-95-6)

VOC Reportable

SARA 311/312 Hazardous

Listed on the United States TSCA (Toxic Substances Control Act) inventory

1,2,4-Trimethylbenzene (95-63-6)

VOC Reportable

SARA 311/312 Hazardous

SARA Section 313 - Emission Reporting: 1.0%

Listed on the United States TSCA (Toxic Substances Control Act) inventory

# Section 16: Other Information

Data Sources:

This document has been prepared in accordance with the SDS requirements of the OSHA Hazard Communication Standard 29 CFR 1910.1200.

GHS Full Text Phrases:

| Code/Abbreviation | Description |
| --- | --- |
| H226 | Flammable liquid and vapor. |
| H315 | Causes skin irritation. |
| H317 | May cause an allergic skin reaction. |
| H320 | Causes eye irritation. |
| H334 | May cause allergy or asthma symptoms or breathing difficulties if inhaled. |
| H335 | May cause respiratory irritation. |
| H351 | Suspected of causing cancer. |
| H402 | Harmful to aquatic life. |
| H412 | Harmful to aquatic life with long lasting effects |

This information is furnished without warranty, representation, inducement or license of any kind, except that it is accurate to the best of Anchor Paint Manufacturing Co., Inc. knowledge or obtained from sources believed by Anchor Paint Manufacturing Co., Inc. to be accurate and Anchor Paint Manufacturing Co., Inc. does not assume any legal responsibility for use or reliance upon same. Customers are encouraged to conduct their own tests.